| Recommended Parts List |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Quantity | Description | Role |  | Estimated Cost (€) |  |
| 1 | PVC water resistant junction box | Housing and protection |  | 4 |  |
| 1 | Arduino Nano (Genuine or Clone) | Micro-controller Unit |  | 10 |  |
| 1 | Arduino Nano screw terminal adapter shield | Connections |  | 5 |  |
| 10 | Female to male Dupont jumper cables | Connections |  | 5 |  |
| 3m | 2x 0.5 mm² Sound cable | Connections |  | 2 |  |
| 1 | 9V USB rechargeable battery | Power |  | 10 |  |
| 1 | 9V battery clip | Connections |  | 1 |  |
| 1 | Step down (9V to 6V) power supply | Power |  | 4 |  |
| 1 | Voltage sensor (Mini breadboard + 2 resistors x 10 MΩ / 0.25 W) | Measurement |  | 3 |  |
| 1 | AM 2301 Temperature and Humidity sensor | Measurement |  | 5 |  |
| 1 | BH1750 Ambient light sensor | Measurement |  | 5 |  |
| 1 | Micro SD card module | Data storage |  | 5 |  |
| 1 | 8 GB (or more) Micro SD card | Data storage |  | 6 |  |
|  |  | ESTIMATED TOTAL COST (€) | < 70 € |  |  |

(*) You should not buy any microcontrollers or sensors with unsoldered connectors unless you are familiar with soldering and you have enough time to spare. The slightly lower price is not worth the trouble.

For example:

Buy this:

Not this :

Dimensions: 150mm X 116mm X 67mm

Wall-mount

IP66

This is a good starter box for our teaching unit. You should void buying a much bigger or smaller box and anything worse than IP54. Make sure that it possesses 6-12 intakes with non threaded rubber (not plastic) cable glands. Only the bottom side glands will be used for all the wiring as they are less prone to water pooling.

IP 66 :

IP stands for International Protection (from electrical junction boxes to smartwatches)

1st 6 means maximum dust protection

2nd 6 means protection against powerful water jets (but not immersion in water)

(Source: https://en.wikipedia.org/wiki/IP_Code)